### Hallowell®

### Utility Bin Shelving Specification

Finishing:  All shelving components to be cleaned and coated after fabrication with a seven stage zinc/iron phosphate solution to inhibit corrosion, followed by a coat of high grade custom blend powder electrostatically sprayed and baked at 350 degrees Fahrenheit for a minimum of 20 minutes to provide a tough durable finish.

Color for all steel components: Shall be 1160 Utility Gray.

Angle Post: Shall be 1.5" x 1.5" dimensions, made from 16 gauge cold rolled steel corner post uprights punched on 1” centers.

Sway Braces:  1" wide 14 gauge steel braces are bolted together at center to form an “X”. Ends are rounded and pierced for mounting to rear and side posts.

Reinforced Shelves: Fabricated from 22 gauge cold rolled steel and triple flanged at all four sides. All four corners are punched to allow shelves to be securely bolted to angle posts. Minimum four nuts and bolts per corner.

One-Year Warranty:  HALLOWELL shelving is covered against all defects in materials and workmanship excluding finish, damage resulting from deliberate destruction and vandalism under this section for a period of one year.